REVERSE ENGINEERING ACTIVITY REPORT

LED Light Bulb 12W (Wipro Garnet 2023 Model)

PART I - STUDYING THE PRODUCT

1. External Features

Physical Construct

The device features an ergonomically shaped, lightweight thermal-plastic and aluminum enclosure composed of a translucent frosted polycarbonate (PC) dome diffuser at the top and a heat-dissipating flame-retardant thermoplastic housing with an internal aluminum sleeve/heatsink. It is designed with a standard cylindrical A67/A65 profile, incorporating a standard B22 Nickel-plated brass bayonet cap (or optional E27 Edison screw cap) at the bottom base for electrical mounting into domestic fixture sockets.

Subsystems Used

Optical & Light-Emitting Subsystem: High-efficiency Surface Mount Device (SMD 2835) LED array matrix mounted on a metal-core printed circuit board (MCPCB) and a polycarbonate diffuser dome.

Thermal Management Subsystem: Extruded/die-cast internal aluminum heat sink body coupled directly to the LED MCPCB substrate via thermal conductive grease/paste.

Power Conversion & Driver Subsystem: Switched-Mode Power Supply (SMPS) constant-current driver PCB, step-down transformer/inductor, input bridge rectifier, high-voltage filter capacitors, and high-voltage surge protection network.

Enclosure & Mechanical Interface: Outer polycarbonate dome, insulating PBT outer shell, inner aluminum heat casing, and B22/E27 nickel-plated base cap.

Description of Subsystems

The outer insulated enclosure acts as a rigid protective chassis housing the internal SMPS driver PCB and providing structural support for the top-mounted Metal Core Printed Circuit Board (MCPCB). The bulb delivers wide-angle omnidirectional illumination by regulating mains AC voltage down to a constant DC current to energize high-efficiency SMD LEDs, dissipating heat via the internal aluminum heatsink and diffusing output through the frosted polycarbonate dome.

Modes of Operation

Continuous AC Constant-Current Mode: Operates continuously when connected to a standard 100V–300V AC power source, supplying constant DC current to deliver steady 1200 lumen output across fluctuations in supply voltage.

2. Purpose of the Product

Provides solid-state, high-efficiency illumination for indoor residential, commercial, and office spaces, converting mains AC electrical energy into uniform, flicker-free cool daylight (6500K) illumination.

3. User Needs Addressed

High Energy Efficiency & Brightness: Delivers high luminous efficacy (~100 lm/W) producing 1200 lumens at only 12W power consumption.

Durability & Extended Lifespan: Rated for 15,000+ operational hours without physical filaments or fragile vacuum glass envelopes.

Voltage Fluctuation & Surge Protection: Built-in protection handling line surges up to 4kV and wide voltage operating ranges (100V–300V AC).

PART II - PRODUCT ORIGIN TO LATEST INVENTIONS

1. Difference in Technology Across Versions

1.1 Early Stage (Incandescent Bulbs)

Utilized a heated tungsten wire filament in a vacuum or inert gas-filled glass bulb. Over 90% of energy was wasted as heat, operating lifespans were limited to ~1,000 hours, and filaments were prone to mechanical breakages.

1.2 First Generation (Compact Fluorescent Lamps - CFL)

Replaced resistive metal filaments with low-pressure mercury-vapor gas discharge generating UV light, converted to visible light via phosphor inner coatings. Improved energy efficiency significantly, but required warmup times, contained toxic mercury, and had fragile glass tubing.

1.3 Second Generation (Standard Constant-Current LED Bulbs - 2023 Wipro Garnet Model)

Introduced high-efficiency surface-mount LED diode arrays driven by solid-state constant-current driver ICs on Metal Core PCBs (MCPCB). Provides instant illumination, long lifespan (15,000 hours), mercury-free construction, and built-in 4kV surge/voltage protection.

1.4 Latest Generation (Smart CCT/RGB Wi-Fi & Matter LED Bulbs)

Incorporates integrated Wi-Fi / Bluetooth System-on-Chip (SoC) microcontrollers alongside multi-channel PWM LED drivers (RGB + Warm White + Cool White). Allows wireless remote dimming, color temperature tuning (2700K–6500K), app control, and voice assistant integration.

2. Current Status - Core Smart Technologies Used in Product

The unit under analysis utilizes a dedicated Constant Current Driver IC (e.g., BP2866/BP2833 series or equivalent single-stage driver). This IC integrates high-voltage MOSFET switches and current-sensing feedback loops on a single silicon die to maintain constant LED output regardless of AC line variations.

3. Engineering Importance of This Evolution

Thermal & Solid-State Reliability: Elimination of glass bulbs, gaseous discharge tubes, and physical filaments reduces mechanical failure while maximizing energy-to-light conversion.

Component-Level Integration: Driver IC integration reduces discrete component count, PCB surface area requirements, assembly overhead, and overall Bill-of-Materials (BOM) cost.

Performance Enhancement: Achieves power factor >0.9, high luminous efficacy (100 lm/W), zero audible hum, and transient voltage suppression protecting internal electronics against mains grid spikes.

PART III - INTERNAL COMPONENTS & WORKING FLOW

1. Introduction of the Product

This tear-down report analyzes a standard Wipro 12W LED Bulb (Garnet 2023 series model). The hardware structure relies on a single-sided paper-phenolic (FR-1/FR-2) driver PCB positioned vertically inside the lower neck and a circular aluminum Metal Core PCB (MCPCB) mounted horizontally under the top diffuser dome.

2. Photograph of the Internal Components of the Product

Figure 1a: Side view of bulb showing outer PBT diffuser body, B22 cap connection, and top frosted PC dome.

Figure 1b: Top-down internal view showing circular MCPCB substrate populated with 12 to 14 SMD 2835 LED packages wired in series/parallel array.

Figure 1c: Close-up view of isolated internal constant-current SMPS driver PCB showing input MOV, bridge rectifier IC, filtering electrolytic capacitors, buck inductor, and main driver IC.

3. Internal Block Diagram (Functional View)

[ AC Mains 240V Input ] ---> [ Surge & MOV Protection ] ---> [ Bridge Rectifier ] ---> [ DC Filter & Storage ] ¦ v [ LED MCPCB Matrix (SMD 2835) ] <--- [ Current Feedback Sense ] <--- [ Constant Current Buck Driver IC ]

4. Internal Subsystems with Detailed Explanation

4.1 Function of the Subsystem

Power Protection & AC-DC Conversion Subsystem: Filters line noise, clamps high-voltage mains transients, and rectifies input AC voltage to high-voltage DC.

Constant Current Regulation Subsystem: Regulates high-voltage DC down to controlled DC current levels, maintaining stable current output to the LED array.

Solid-State Light Array Subsystem: Converts regulated DC current into visible light via semiconductor electroluminescence.

4.2 Working Principle

220V–240V AC power enters via the B22 cap lead wires, passing through a safety fuse resistor and a Metal Oxide Varistor (MOV) for spike suppression. A full-wave bridge rectifier converts AC to DC, filtered by electrolytic capacitors. The buck-converter LED driver IC regulates current across an energy-storage inductor, delivering constant milliampere current to the SMD 2835 LEDs mounted on the high-thermal-conductivity MCPCB.

4.3 Characteristics

Operating Voltage: 100V – 300V AC at 50 Hz.

Luminous Output: 1200 lumens (Cool Daylight 6500K).

Power Factor: >0.9; High Luminous Efficacy (100 lm/W).

4.4 Role / Engineering Justification

Deploying a single-chip constant-current buck driver eliminates complex multi-stage transformer circuitry, reducing driver footprint, lowering overall unit cost, improving power factor, and providing thermal foldback protection against over-temperature conditions.

5. Complete Working Flow (Step-By-Step)

1. Power Input: Inserting the bulb base into an active fixture socket supplies 220–240V AC across the base contact pins.

2. Surge Protection: Input current flows through a series flameproof fuse resistor and parallel MOV to absorb potential high-voltage grid transients up to 4kV.

3. Full-Wave Rectification: A single-phase bridge rectifier converts AC line voltage into pulsating direct current (DC).

4. DC Filtering: High-voltage electrolytic capacitors smooth output voltage ripple to establish a stable high-voltage DC bus line.

5. Current Regulation: The constant-current driver IC monitors current feedback via external sense resistors and toggles its internal high-voltage MOSFET switch, driving the power inductor in a buck topology.

6. Photonic Emission: Continuous regulated DC current flows through series-connected SMD 2835 LEDs on the aluminum MCPCB, emitting light through the translucent PC diffuser dome.

6. Components List - Bill of Materials (BOM)

| Ref. Des. | Component Name / Description | Package / Type | Qty |
| --- | --- | --- | --- |
| U1 | Constant Current LED Driver IC (e.g., BP2866) | SOP-8 / SOT33-7 | 1 |
| BR1 | Full-Wave Bridge Rectifier (e.g., MB10F) | 4-Pin SMD / ABS | 1 |
| MOV1 | Metal Oxide Varistor (470V / 4kV Surge Protection) | 7mm Radial Through-Hole | 1 |
| FR1 | Wirewound Fuse Resistor (10Ω / 1W–2W) | Axial Through-Hole | 1 |
| L1 | Power Buck Drum Inductor | Radial Shielded / Unshielded | 1 |
| C1, C2 | High-Voltage Input Filter Capacitors (400V 4.7µF–10µF) | Radial Electrolytic | 2 |
| C3 | Driver IC VCC Decoupling Capacitor (50V 10µF) | Radial / SMD Ceramic | 1 |
| RS1, RS2 | Current Sensing Resistors (SMD 1206 Precision) | SMD Surface Mount | 2 |
| MCPCB1 | Aluminum Metal Core PCB with 12W SMD 2835 LEDs | Circular Metal-Core PCB | 1 |

7. Component Ratings & Specifications

| Component | Parameter / Marking | Value / Specification |
| --- | --- | --- |
| MCPCB Substrate | Material / Thermal Conductivity | Aluminum Core Board, 1.2mm thickness, >1.5 W/mK |
| LED Driver IC (U1) | SOP-8 Package | Internal 500V Power MOSFET, Constant Current Control, Thermal Foldback |
| SMD LEDs | 2835 High-Efficacy LED Chips | Forward Voltage ~9V–18V DC/chip; Cool Daylight 6500K, CRI >80 |
| Surge Protection (MOV1) | 7D471K / 4KV | Clamping Voltage ~470V; 4kV Surge Endurance Rating |
| Base Connector | B22d / E27 Cap | Nickel-plated brass / zinc-coated alloy |

8. Component Function Table

| Component Name | Function | Specifications |
| --- | --- | --- |
| LED Driver IC (U1) | Monitors current loop via sense resistors, switching internal MOSFET to deliver steady current output to LED matrix. | Integrated SOP-8 constant current control IC. |
| Bridge Rectifier (BR1) | Converts bi-directional AC mains voltage into single-direction pulsating DC. | 1000V 1A SMD Package (MB10F or equivalent). |
| Metal Oxide Varistor (MOV1) | Suppresses high-voltage grid surges by dropping resistance during overvoltage events. | 4kV surge absorption rating. |
| Fuse Resistor (FR1) | Limits inrush current upon power-up and open-circuits under catastrophic failure conditions. | 10 Ohm 1W-2W flame-proof axial lead resistor. |
| Power Inductor (L1) | Stores magnetic energy during MOSFET conduction cycles and discharges to maintain steady current flow. | High-saturation drum core radial inductor. |
| Filter Capacitors (C1/C2) | Smooth out high-voltage DC ripple behind the bridge rectifier. | 400V 105°C rated high-temperature aluminum electrolytic. |